| Model: | Y6 |
| --- | --- |
| APP: | Yiiot |
| System structure: | Embedded Linux system, ARM chip structure |
| Chip: | AK3918E |
| Resolution: | 2.0MP |
| Sensor Resolution: | 1/4"GC1054、SC1245 1920*1080 |
| Lens: | 3.6MM |
| View angle: | 80° |
| Pan-tilt: | Rotates Horizontal:355°Vertical：90° |
| Preset point quantity: | 6 |
| Video compression standard: | H.264/20FPS |
| Video format: | PAL |
| Minimum illumination: | 0.01Lux@（F2.0，VGC ON),O.Luxwith IR |
| Electronic shutter: | Auto |
| Backlight compensation: | Support |
| Noise reduction: | 2D、3D |
| Led quantity: | 4PCS infrared LED、4PCS White LED |
| Network: | WIFI wireless transmission (support IEEE802.11b/g/n wireless protocol).without RJ45 port |
| Network connection: | WIFI, AP Hotspot |
| Night vision | IR-CUT switch Automatic，about 5-8meters (It varies from environment) |
| Audio: | Built-in microphone and speaker, support two-way audio and real-time transmission. ADPCM audio compression standard, self-adapt to code stream |
| Network protocol: | TCP/IP、UDP、RTSP、HTTPDDNS、DHCP、FTP、NTP |
| Alarm: | ①Motion detection and picture push ②AI Human intrusion Detection (optional) |
| Storage: | TF Card（Max 128G）；Cloud storage /Cloud disk (optional) |
| Power input: | 5V/1.6A |
| Work environment: | Working temperature:-10℃ ~ + 50℃ Working Humidity: ≤95%RH |